CNC LATHE
PROGRAMMING & MACHINING
CASE MANUAL

Case drawings, programming logic, representative G-code, cycle interpretation and shop-floor review notes.

| G71 / G70 | G72 / G73 | G74 / G75 | G76 / G92 |
| --- | --- | --- | --- |
| G02 / G03 | G96 / G97 | C-Axis | Live Tooling |

How to Use This Manual

Each case follows a compact structure: source drawing, machining objective, representative program, program logic and practical review points. The source programs are teaching examples; controller syntax, M-codes, offsets, compensation and safe positions must be validated on the actual machine.

Core Cycle Reference

• G71 - longitudinal rough turning

• G70 - finishing a P/Q contour

• G72 - facing roughing

• G73 - pattern-repeating roughing

• G74 / G75 - grooving

• G76 / G92 - threading

• G96 / G97 - constant surface speed / fixed rpm

• G02 / G03 - circular interpolation

CASE 01  |  G71 Arc Groove

G71 ROUGH TURNING

|  | Machining Objective Rough a concave arc-groove contour and leave stock for finishing Program Logic The program uses G71 rough turning to generate the feature shown in the drawing. Review the start position, cutting direction, stock allowance, feed mode and safe retract before running the cycle. |
| --- | --- |

Representative CNC Program

T0101 S600 M03 G0 X52 Z5 G71 U1.5 R2 G71 P1 Q2 U0.5 W0.5 F0.2 N1 G0 X50 G1 Z-6.5 G3 X50 Z-18.5 R6.5 N2 G1 X52 G0 X150 Z150 M30

Key Review Points

• Verify work zero, raw-stock size and chuck/jaw clearance.

• Confirm tool number, offset, insert geometry and spindle/feed mode.

• Check every rapid move and the final retract position.

• Confirm P/Q contour validity and finishing allowance on the actual controller.

CASE 02  |  G92 Thread Cutting Loop

G92 THREADING

|  | Machining Objective Cut an external thread through repeated G92 passes at progressively smaller X diameters Program Logic The program uses G92 threading to generate the feature shown in the drawing. Review the start position, cutting direction, stock allowance, feed mode and safe retract before running the cycle. |
| --- | --- |

Representative CNC Program

T0101 S600 M03 G99 G0 X32 Z4 G92 X29.3 Z-28 F2 X28.9 X28.7 X28.5 X28.1 X27.8 G0 X150 Z150 M30

Key Review Points

• Verify work zero, raw-stock size and chuck/jaw clearance.

• Confirm tool number, offset, insert geometry and spindle/feed mode.

• Check every rapid move and the final retract position.

• Confirm thread pitch, final diameter, infeed depth and run-out clearance.

CASE 03  |  G71 Comprehensive Profile

G71 + G70

|  | Machining Objective Rough a stepped multi-radius profile, change tools, then finish the same P/Q contour Program Logic The program uses G71 + G70 to generate the feature shown in the drawing. Review the start position, cutting direction, stock allowance, feed mode and safe retract before running the cycle. |
| --- | --- |

Representative CNC Program

T0101 S600 M03 G99 G0 X76 Z0 G71 U1.5 R2 G71 P1 Q2 U0.5 W0.5 F0.2 N1 ... final contour ... N2 X76 T0202 S1600 M03 G70 P1 Q2 M30

Key Review Points

• Verify work zero, raw-stock size and chuck/jaw clearance.

• Confirm tool number, offset, insert geometry and spindle/feed mode.

• Check every rapid move and the final retract position.

• Confirm P/Q contour validity and finishing allowance on the actual controller.

CASE 04  |  G71 Stepped Profile

G71 CONTOUR ROUGHING

|  | Machining Objective Apply G71 to a multi-diameter external profile with shoulders and a radius transition Program Logic The program uses G71 contour roughing to generate the feature shown in the drawing. Review the start position, cutting direction, stock allowance, feed mode and safe retract before running the cycle. |
| --- | --- |

Representative CNC Program

T0101 S600 M03 G0 X76 Z0 G71 U1.5 R2 G71 P1 Q2 U0.5 W0.5 F0.2 N1 G0 X20 G1 ... G3 ... R10 N2 X76 G0 X150 Z150 M30

Key Review Points

• Verify work zero, raw-stock size and chuck/jaw clearance.

• Confirm tool number, offset, insert geometry and spindle/feed mode.

• Check every rapid move and the final retract position.

• Confirm P/Q contour validity and finishing allowance on the actual controller.

CASE 05  |  U-Shaped Groove

G71 / G70

|  | Machining Objective Machine a U-shaped groove by embedding an arc in the P/Q contour Program Logic The program uses G71 / G70 to generate the feature shown in the drawing. Review the start position, cutting direction, stock allowance, feed mode and safe retract before running the cycle. |
| --- | --- |

Representative CNC Program

T0101 M03 S800 G0 X43.5 Z-5.5 G71 U1 R0.5 F0.2 G71 P1 Q2 U0.5 W0 N1 G0 X43 W0 G1 X40 Z-5.5 F0.15 G2 X40 Z-18.5 R6.5 N2 G1 X43 G70 P1 Q2 M30

Key Review Points

• Verify work zero, raw-stock size and chuck/jaw clearance.

• Confirm tool number, offset, insert geometry and spindle/feed mode.

• Check every rapid move and the final retract position.

• Confirm P/Q contour validity and finishing allowance on the actual controller.

CASE 06  |  Gourd-Shaped Multi-Radius Profile

COMPLEX CONTOUR TURNING

|  | Machining Objective Combine several G02/G03 arcs to generate a decorative profile Program Logic The program uses Complex contour turning to generate the feature shown in the drawing. Review the start position, cutting direction, stock allowance, feed mode and safe retract before running the cycle. |
| --- | --- |

Representative CNC Program

T0101 M03 S1000 G99 G0 X35.5 Z1 G71 U0.8 R0.4 G71 P1 Q2 U0.2 W0 N1 G0 X0 G1 Z0 F0.15 G03 ... G02 ... N2 X35.5 G70 P1 Q2 M30

Key Review Points

• Verify work zero, raw-stock size and chuck/jaw clearance.

• Confirm tool number, offset, insert geometry and spindle/feed mode.

• Check every rapid move and the final retract position.

CASE 07  |  Constant Surface Speed Profile

G96 / G97

|  | Machining Objective Use constant surface speed while the cutting diameter changes Program Logic The program uses G96 / G97 to generate the feature shown in the drawing. Review the start position, cutting direction, stock allowance, feed mode and safe retract before running the cycle. |
| --- | --- |

Representative CNC Program

G92 X40 Z5 M03 S400 G96 S80 G00 X0 G01 Z0 F60 G03 U24 W-24 R15 G02 X26 Z-31 R5 G01 Z-40 X40 Z5 G97 S300 M30

Key Review Points

• Verify work zero, raw-stock size and chuck/jaw clearance.

• Confirm tool number, offset, insert geometry and spindle/feed mode.

• Check every rapid move and the final retract position.

CASE 08  |  G71 Tapered Profile

G71 + G70

|  | Machining Objective Rough and finish a tapered profile containing small radius transitions Program Logic The program uses G71 + G70 to generate the feature shown in the drawing. Review the start position, cutting direction, stock allowance, feed mode and safe retract before running the cycle. |
| --- | --- |

Representative CNC Program

G99 T0101 G97 S600 M03 G0 X31 Z0 G71 U1.5 R0.5 G71 P1 Q2 U0.5 W0.2 F0.2 N1 G0 X0 G3 X6 Z-3 R3 F0.1 G2 X12 Z-6 R3 G1 X20 Z-20 N2 X31 G70 P1 Q2 M30

Key Review Points

• Verify work zero, raw-stock size and chuck/jaw clearance.

• Confirm tool number, offset, insert geometry and spindle/feed mode.

• Check every rapid move and the final retract position.

• Confirm P/Q contour validity and finishing allowance on the actual controller.

CASE 09  |  Concave / Convex Arc Turning

G02 / G03

|  | Machining Objective Practice clockwise and counterclockwise circular interpolation in the lathe X-Z plane Program Logic The program uses G02 / G03 to generate the feature shown in the drawing. Review the start position, cutting direction, stock allowance, feed mode and safe retract before running the cycle. |
| --- | --- |

Representative CNC Program

T0101 M03 S1000 G99 G0 X28 Z1 G01 X30 Z0 F0.15 G03 X40 Z-5 R5 G01 Z-35 G02 X50 Z-40 R5 G0 X100 Z100 M30

Key Review Points

• Verify work zero, raw-stock size and chuck/jaw clearance.

• Confirm tool number, offset, insert geometry and spindle/feed mode.

• Check every rapid move and the final retract position.

CASE 10  |  Pulley Groove Profile

G75 + PROFILE MOVES

|  | Machining Objective Machine a pulley-style groove using grooving and direct contour moves Program Logic The program uses G75 + profile moves to generate the feature shown in the drawing. Review the start position, cutting direction, stock allowance, feed mode and safe retract before running the cycle. |
| --- | --- |

Representative CNC Program

T0101 M03 S800 G99 G0 X41 Z-15 G75 R0.6 G75 X22 Z-17 P2 Q3.50 F0.1 G0 X42 Z-21 G1 X40 F0.1 X22 Z-15 Z-17 G0 X100 Z100 M30

Key Review Points

• Verify work zero, raw-stock size and chuck/jaw clearance.

• Confirm tool number, offset, insert geometry and spindle/feed mode.

• Check every rapid move and the final retract position.

CASE 11  |  G90 vs. G91 Coordinate Modes

COORDINATE PROGRAMMING

|  | Machining Objective Understand absolute and incremental coordinate programming Program Logic The program uses Coordinate programming to generate the feature shown in the drawing. Review the start position, cutting direction, stock allowance, feed mode and safe retract before running the cycle. |
| --- | --- |

Representative CNC Program

G90 X80 Y80 G91 X40 Y40 G90 X40 Y40 G91 X-40 Y-40

Key Review Points

• Verify work zero, raw-stock size and chuck/jaw clearance.

• Confirm tool number, offset, insert geometry and spindle/feed mode.

• Check every rapid move and the final retract position.

CASE 12  |  G02 Radius Profile

G02 ARC PROGRAMMING

|  | Machining Objective Turn an external R8 transition and finish the contour Program Logic The program uses G02 arc programming to generate the feature shown in the drawing. Review the start position, cutting direction, stock allowance, feed mode and safe retract before running the cycle. |
| --- | --- |

Representative CNC Program

T0101 M03 S1000 G0 X58 Z1 G71 U1 R0.4 G71 P1 Q2 U0.4 W0.1 F0.3 N1 G0 X40 G01 Z-42 F0.15 N2 G02 X56 Z-50 R8 F0.15 G70 P1 Q2 M30

Key Review Points

• Verify work zero, raw-stock size and chuck/jaw clearance.

• Confirm tool number, offset, insert geometry and spindle/feed mode.

• Check every rapid move and the final retract position.

CASE 13  |  Live-Tool Hex Milling

C-AXIS LIVE TOOLING

|  | Machining Objective Machine six flats with a driven tool and C-axis interpolation/indexing Program Logic The program uses C-axis live tooling to generate the feature shown in the drawing. Review the start position, cutting direction, stock allowance, feed mode and safe retract before running the cycle. |
| --- | --- |

Representative CNC Program

T0101 G17 M23 G0 G54 X24.928 Z25 C0 S2000 M52 G112 ... indexed milling moves ... G40 X-60.928 G0 Z25 G113 M55 M30

Key Review Points

• Verify work zero, raw-stock size and chuck/jaw clearance.

• Confirm tool number, offset, insert geometry and spindle/feed mode.

• Check every rapid move and the final retract position.

• Machine-builder M-codes and C-axis interlocks must be verified on the actual turning center.

CASE 14  |  Face Drilling with C-Axis

LIVE-TOOL DRILLING

|  | Machining Objective Drill an indexed bolt-hole pattern on the end face Program Logic The program uses Live-tool drilling to generate the feature shown in the drawing. Review the start position, cutting direction, stock allowance, feed mode and safe retract before running the cycle. |
| --- | --- |

Representative CNC Program

T0101 G17 M23 G0 G54 X35 Z10 C0 S1600 M52 G83 Z-15 R2 Q3 F120 C60 C120 C180 C240 C300 G80 M55 M30

Key Review Points

• Verify work zero, raw-stock size and chuck/jaw clearance.

• Confirm tool number, offset, insert geometry and spindle/feed mode.

• Check every rapid move and the final retract position.

• Machine-builder M-codes and C-axis interlocks must be verified on the actual turning center.

CASE 15  |  G73 Profile Roughing

G73 PATTERN REPEATING

|  | Machining Objective Rough a near-net-shape blank by repeating offset versions of the final contour Program Logic The program uses G73 pattern repeating to generate the feature shown in the drawing. Review the start position, cutting direction, stock allowance, feed mode and safe retract before running the cycle. |
| --- | --- |

Representative CNC Program

M03 S600 T0101 G0 X87 Z2 G73 U4 W4 R14 G73 P1 Q2 U0.5 W0.5 F0.2 N1 G0 X20 G01 Z-5 G03 X30 Z-5 R5 ... N2 X87 G0 X150 Z150 M30

Key Review Points

• Verify work zero, raw-stock size and chuck/jaw clearance.

• Confirm tool number, offset, insert geometry and spindle/feed mode.

• Check every rapid move and the final retract position.

• Confirm P/Q contour validity and finishing allowance on the actual controller.

CASE 16  |  G72 Facing Roughing

G72 FACING CYCLE

|  | Machining Objective Remove stock primarily in the facing direction Program Logic The program uses G72 facing cycle to generate the feature shown in the drawing. Review the start position, cutting direction, stock allowance, feed mode and safe retract before running the cycle. |
| --- | --- |

Representative CNC Program

T0101 S800 M03 G0 X112 Z2 G72 W1.5 R1 F0.2 G72 P1 Q2 U0.5 W0.5 N1 G00 Z-15 G01 X70 Z-5 X30 Z1 N2 X112 G0 X150 Z150 M30

Key Review Points

• Verify work zero, raw-stock size and chuck/jaw clearance.

• Confirm tool number, offset, insert geometry and spindle/feed mode.

• Check every rapid move and the final retract position.

• Confirm P/Q contour validity and finishing allowance on the actual controller.

CASE 17  |  G74 End-Face Grooving

G74 GROOVING

|  | Machining Objective Cut an end-face groove with repeated controlled infeed Program Logic The program uses G74 grooving to generate the feature shown in the drawing. Review the start position, cutting direction, stock allowance, feed mode and safe retract before running the cycle. |
| --- | --- |

Representative CNC Program

T0101 S800 M03 G0 X57.5 Z2 G74 R0 G74 X50 Z-4.8 P3.5 Q0 F0.15 G0 Z2 X150 Z150 M05 M30

Key Review Points

• Verify work zero, raw-stock size and chuck/jaw clearance.

• Confirm tool number, offset, insert geometry and spindle/feed mode.

• Check every rapid move and the final retract position.

CASE 18  |  Indexed End-Face Drilling

C-AXIS DRILLING

|  | Machining Objective Repeat a drilling cycle at successive angular positions Program Logic The program uses C-axis drilling to generate the feature shown in the drawing. Review the start position, cutting direction, stock allowance, feed mode and safe retract before running the cycle. |
| --- | --- |

Representative CNC Program

T0101 G17 M23 G0 G54 X35 Z10 C0 S1600 M52 G83 Z-15 R2 Q3 F120 C60 C120 C180 C240 C300 G80 M55 M30

Key Review Points

• Verify work zero, raw-stock size and chuck/jaw clearance.

• Confirm tool number, offset, insert geometry and spindle/feed mode.

• Check every rapid move and the final retract position.

• Machine-builder M-codes and C-axis interlocks must be verified on the actual turning center.

CASE 19  |  G75 Radial Grooving

G75 GROOVING

|  | Machining Objective Machine a radial groove using pecking and axial stepping Program Logic The program uses G75 grooving to generate the feature shown in the drawing. Review the start position, cutting direction, stock allowance, feed mode and safe retract before running the cycle. |
| --- | --- |

Representative CNC Program

T0101 S600 M03 G99 G0 X84 Z-21.5 G75 R3 G75 X60.5 Z-34.5 P3000 Q500 F0.15 G0 X84 Z-21 G1 X60 F0.1 Z-25 G0 X84 G0 X150 Z150 M30

Key Review Points

• Verify work zero, raw-stock size and chuck/jaw clearance.

• Confirm tool number, offset, insert geometry and spindle/feed mode.

• Check every rapid move and the final retract position.

CASE 20  |  G76 Multi-Pass Threading

G76 THREADING

|  | Machining Objective Cut a thread using a two-block multi-pass threading cycle Program Logic The program uses G76 threading to generate the feature shown in the drawing. Review the start position, cutting direction, stock allowance, feed mode and safe retract before running the cycle. |
| --- | --- |

Representative CNC Program

S600 M03 G99 T0101 G0 X54 Z2 G76 P010060 Q100 R100 G76 X45.4 Z-32 P1300 Q500 R0 F2 G0 X150 Z150 M05 M30

Key Review Points

• Verify work zero, raw-stock size and chuck/jaw clearance.

• Confirm tool number, offset, insert geometry and spindle/feed mode.

• Check every rapid move and the final retract position.

• Confirm thread pitch, final diameter, infeed depth and run-out clearance.

Program Prove-Out Checklist

• Work zero and Z-face reference verified

• Raw-stock diameter and stick-out match the program

• Chuck and jaw envelope checked against all Z positions

• Tool numbers, offsets and insert orientation confirmed

• Spindle direction, speed mode and speed limit confirmed

• Feed mode and programmed feeds confirmed

• Thread pitch, groove width and drilling depth independently checked

• Rapid moves reviewed for collision risk

• Dry run completed with reduced rapid override

• First part inspected before continuous production

Important: These programs are training examples derived from the supplied case material. CNC cycle formats, M-codes, compensation behavior and safety requirements vary by controller and machine builder. Validate every program on the specific machine before production.